25G NIC Bonding – Validation & Performance Test Report

Server A (172.16.12.131)  ↔  Server B (172.16.13.108)

Prepared by: Shailendra Rajput      |      Date: 28 May 2026

## 1.  Test Objective

| Purpose | Validate and benchmark dual-port 25G NIC performance with and without NIC bonding (link aggregation) between two servers. |
| --- | --- |
| Test Focus | Throughput and latency of a single 25G link vs a two-link bond, and comparison of Linux bonding modes. |

## 2.  Common Test Environment

| Operating System | Ubuntu 22.04.4 LTS |
| --- | --- |
| Kernel | 5.15.0 (Server A: -179-generic, Server B: -94-generic) |
| Test Tools | iperf3 3.9 (throughput), qperf (latency), ethtool, ping |
| NIC Under Test | Broadcom BCM57414 NetXtreme-E, dual-port 25GbE (driver bnxt_en) |
| Optics / Cable | SFP28 25G-SR fibre (Cisco-OEM); direct back-to-back (no switch) |
| Link Tuning | MTU 9000 (jumbo), RX/TX rings 2047, flow control ON, NUMA-pinned iperf3 |
| Topology | Two 25G links per server bonded; direct server-to-server fibre |
| Test Date | 28 May 2026 |

## 3.  Server A – System Under Test

| Management IP | 172.16.12.131 |
| --- | --- |
| CPU | AMD EPYC 9124 (2 sockets, 16-core, 64 threads) |
| Memory | 251 GiB |
| Role | Kubernetes node (Calico overlay) – production |
| Management NIC | enp33s0f0 (1 GbE, igb) – not under test |
| 25G NIC 1 | Broadcom BCM57414 @ PCIe 0000:81:00 – x4 link (31.5 Gb/s) |
| 25G NIC 2 | Broadcom BCM57414 @ PCIe 0000:c1:00 – x8 link (63 Gb/s) |

## 4.  Server B – System Under Test

| Management IP | 172.16.13.108 |
| --- | --- |
| CPU | AMD EPYC 9745 (2 sockets, 128-core, 512 threads) |
| Memory | 1.5 TiB |
| Management NIC | enp84s0f1 (1 GbE, igb) – not under test |
| 25G NIC 1 | Broadcom BCM57414 @ PCIe 0000:61:00 – x8 link (63 Gb/s) |
| 25G NIC 2 | Broadcom BCM57414 @ PCIe 0000:91:00 – x8 link (63 Gb/s) |

## 5.  Test Methodology

- Each 25G port was linked back-to-back over fibre and verified at 25 Gb/s with jumbo frames (0% loss at 8972-byte DF ping).

- Baseline measured on a single 25G link (no bond), both directions, with 1 and 8 parallel TCP streams.

- Bond configured via Linux iproute2 (non-persistent) across two 25G ports per server. Modes tested: balance-rr (mode 0) and balance-xor (mode 2, layer3+4 hash).

- Throughput taken from iperf3 sender bitrate; latency from qperf (TCP/UDP) and ICMP ping; TCP retransmissions recorded as a link-quality indicator.

## 6.  Results – Baseline (Single 25G Link, No Bond)

| Test Case | Streams | Throughput | Retransmits |
| --- | --- | --- | --- |
| Single link A → B | 1 | 16.9 Gbit/s | 0 |
| Single link A → B | 8 | 17.5 Gbit/s | ~14 |
| Single link B → A | 8 | 24.2 Gbit/s | 0 |
| Two links simultaneously (independent) | 8 + 8 | 32.0 Gbit/s (sum) | low |

qperf single-stream A→B = 17.3 Gbit/s (confirms iperf3). Clean, near-zero loss with jumbo frames.

## 7.  Results – Bond Mode: balance-rr (mode 0)

| Test Case | Streams | Throughput | Retransmits |
| --- | --- | --- | --- |
| Bond A → B | 1 | 16.0 Gbit/s | 63 |
| Bond A → B | 8 | 17.4 Gbit/s | 17 |
| Bond B → A | 1 | 19.9 Gbit/s | 41,004 |
| Bond B → A | 8 | 25.5 Gbit/s | 4,373 |

Round-robin striping causes severe packet reordering (tens of thousands of retransmissions) and no aggregation benefit. Not suitable for TCP.

## 8.  Results – Bond Mode: balance-xor (mode 2, layer3+4)

| Test Case | Streams | Throughput | Retransmits |
| --- | --- | --- | --- |
| Bond A → B | 1 | 16.9 Gbit/s | 0 |
| Bond A → B | 8 | 18.1 Gbit/s | 7 |
| Bond A → B | 16 | 22.2 Gbit/s | 1 |
| Bond B → A | 8 | 15.2 Gbit/s | 32,212 |
| Bond B → A | 16 | 9.9 Gbit/s | 21,370 |

A→B aggregates cleanly up to ~22 Gbit/s with many flows. B→A degrades because both of Server A's bonded ports share a single PCIe x4 slot (31.5 Gb/s), causing receive-side packet loss.

## 9.  Latency

| Metric | No Bond | Bond (balance-xor) |
| --- | --- | --- |
| TCP latency (qperf, one-way) | 18 µs | 24 µs |
| UDP latency (qperf, one-way) | 19 µs | 22 µs |
| ICMP round-trip (ping) | 0.45 ms | 0.52 ms |

## 10.  Throughput Comparison

Figure 1 – Peak observed throughput by configuration and direction (iperf3, MTU 9000).

## 11.  Key Findings

- A single 25G link is healthy: ~24 Gbit/s (B→A), ~17 Gbit/s (A→B), ~18 µs latency, near-zero loss with jumbo frames.

- Directional asymmetry: A→B consistently ~17 Gbit/s vs B→A ~24 Gbit/s per link (receive-path / CPU contention on the busy Kubernetes node A).

- Bonding did not cleanly double throughput, due to two host constraints: (a) one of Server A's 25G cards is in a PCIe x4 slot (31.5 Gb/s), and the only stable two-link bond forced both A ports onto that card – capping aggregate ~31G and causing RX loss on B→A; (b) several inter-server fibre links were marginal/unidirectional, forcing a sub-optimal port pairing (RS-FEC off, mixed optics).

- balance-rr (round-robin) is unsuitable for TCP – heavy reordering and tens of thousands of retransmissions with no real gain.

- Repeated bond reconfiguration stressed the bnxt driver and briefly hung Server B's network stack (self-recovered, no reboot).

## 12.  Recommendations

- Move Server A's 25G card from the PCIe x4 slot into an x8 slot (or bond one port per card across two x8 cards) – the single biggest blocker to real 2×25G aggregation.

- Use matching 25G-SR optics on both ends of every link, clean/reseat the LC connectors, and enable RS-FEC (ethtool --set-fec <dev> encoding rs).

- For production bonding, prefer 802.3ad (LACP) via a switch over back-to-back, and persist the configuration in netplan once links are stable.

- Use balance-xor or 802.3ad (flow hashing) for TCP – never balance-rr.

- Expected after the above: ~45–48 Gbit/s multi-flow aggregate on a 2×25G bond.

## 13.  Final Conclusion

| NIC Health | Both 25G NICs link at 25 Gb/s; jumbo frames stable, 0% packet loss |
| --- | --- |
| Single-Link Performance | PASS – up to ~24 Gbit/s, ~18 µs latency |
| Bonding (functional) | PASS – bond forms and carries traffic in both modes |
| Bonding (aggregation) | LIMITED – capped by host PCIe x4 slot and marginal optics (not a NIC fault) |
| Recommended Mode | balance-xor / 802.3ad (layer3+4); avoid balance-rr |
| Final Status | PASS WITH PLATFORM LIMITATIONS |

## 14.  Document Control

| Prepared By | Shailendra Rajput |
| --- | --- |
| Date | 28 May 2026 |
| Document | 25G NIC Bonding – Validation & Performance Test Report |
| Classification | Confidential |